3.	Twenty-five hundredths (0.25) grains of hydrogen sulfide per one hundred (100) cubic feet of gas; and

4.	Twenty  (20) grains of total sulfur per one hundred (100) cubic feet of gas.

The Total Heating Value of the gas shall be determined by taking samples of the gas at the point(s) of receipt at such reasonable times as may be designated by Company.  The Btu content per cubic foot shall be determined by an accepted type of calorimeter or other suitable instrument for a cubic foot of gas at a temperature of sixty (60) degrees Fahrenheit when saturated with water vapor and at a pressure of 14.73 psia.  The Btu determination designated by Company shall be made by Company at its expense.  Any additional Btu determinations requested by Customer shall be at Customer's expense.

Customer's gas delivered to Company shall have a total heating value of not less than nine hundred and sixty seven (967) Btu per standard cubic foot, and shall have a Utilization Factor of one thousand, three hundred (1,300) plus or minus six percent (6%).  The Utilization Factor, as used herein, shall be the Btu content per cubic foot of the gas divided by the square root of the specific gravity of the gas.  However, Company shall not be obligated to accept gas which it believes may adversely affect the standard of public utility service offered by Company, or gas which it believes may adversely affect the operation of the gas-burning equipment of its customers.

If any gas delivered hereunder fails to meet the quality specifications set forth herein, Company may, at any time, elect to refuse to accept all or any portions of such gas until Customer brings the gas into conformity with such specifications.

7. AUTHORIZED DAILY VOLUME

Customer's Authorized Daily Volume on any day consists of the sum of Customer's transported volumes (as determined herein) plus any Backup Service for which Customer has contracted, plus any additional volumes that Company, in its sole discretion, authorizes Customer to use on that day.  Delivery of Customer's Authorized Daily Volume is firm, with no planned interruptions, except as provided in Part 8 hereof.  Consumption at Customer's facility in excess of the Authorized Daily Volume is interruptible service, and upon notice to Customer, Company may require Customer to reduce consumption to Customer's Authorized Daily Volume whenever Company, in its discretion, deems necessary to do so.  Company may, at its option, require such reductions in consumption by Large General Transportation Service Customers prior to imposing similar reductions on Small General and General Transportation Service Customers.  The Authorized Daily Volume for an Aggregation Pool shall equal the sum of the Authorized Daily Volumes (including Backup Service) for each Customer in the Aggregation Pool.